LXSG-15C

Multi-jet, vane wheel, dry-dial water meter

Application:

● Measuring the total volume of cold water passing through the pipeline.

NOTE:

● This type of water meter can be used for a remote reading transmission system as

equipped with a built-in-sensor.

Feature:

● Multi-jet, dry-dial; Magnetic drive;

● Vacuum sealed register ensures the dial keep free from fog and frost, Keep the reading

clear in a long term service;

● Removable measuring unit,easy installation and maintenance, Antimagnetic function;

● Measuring high flowtate with low head loss,inner and outside adjustment is optional;

● Remote transmission device can be added upon request:

such as Reed Swith or Hall, Pulse Output:1000L/P、100L/P、10L/P、1L/P;

● Technical data conform to ISO4064 standard Class C

Working condition:

● Water temperature: T30　T50

● Water pressure class: MAP16　MAP10

Maximum permissible errors:

● in the lower zone from qmin inclusive up to but excluding qt is ±5%.

● in the upper zone from qt inclusive up to and including qs is ±2%;

Main Technical Specification

| LXSG-15 MULTI-JET, VANE WHEEL, DRY-DIAL WATER METER |  |  |  |
| --- | --- | --- | --- |
| Code | Description | Material | QTY |
| 1 | Lid | HDPE（ABS） | 1 |
| 2 | Hinge Pin | Stainless steel | 1 |
| 3 | Cap | Brass | 1 |
| 4 | Sliding Gasket | PP | 1 |
| 5 | Tempered Glass | PC | 1 |
| 6 | Lining ring | Plastic | 1 |
| 7 | Register | Assembly | 1 |
| 8 | Central Gear | Component | 1 |
| 9 | Magnetic Shield Ring | Iron | 1 |
| 10 | Gearing box | ABS Plastic | 1 |
| 11 | Seal Ring | Silicon Rubber | 1 |
| 12 | Turbine Component | Component | 1 |
| 13 | Turbine Box | HIPS | 1 |
| 14 | Adjusting Gasket | Nylon | 1 |
| 15 | Adjusting Plug | Brass | 1 |
| 16 | Adjusting Screw | POM | 1 |
| 17 | Body | Brass | 1 |
| 18 | Coupling Gasket | Rubber | 2 |
| 19 | Coupling Nut | Brass | 2 |
| 20 | Inlet Strainer | PP | 2 |
| 21 | Coupling Tail Piece | Brass | 2 |